Capstone Assumptions

- Design rules: prices live only in the parameter file; untested combinations are refused; the interface performs no arithmetic; every output traces to a measured CCT value or a dated price.

- Portion scaling

- Tarriff band interaction

Data files

cct_lookup.csv — 46 appliance–dish records, each with fuel type, food weight, fuel mass (non-electric), energy in kWh, cooking time in minutes, number of tests, ease-of-use and taste scores, and a notes column recording every correction applied to that row.

clean_cct_data.py — the cleaning script, which is also your provenance record. Key decisions it encodes: all kWh-equivalents for fuel appliances are recomputed from raw fuel mass × calorific value (the stored column had errors up to 14×); all times are recomputed from start/end timestamps, resolving the contradictory time columns; the ethanol–chips 1015 g outlier is excluded per the study's own >15% divergence protocol; and rows where energy tests still diverge >15% carry a caution flag.

Spot-checks against the report: beans (EPC 0.32, induction 0.82, charcoal 3.94), rice–ethanol now 0.43 instead of 6.117, beef–ICS now 2.09 instead of 13.28, induction–chapati 0.025 matching the report's 0.03.

One item needs your judgment before the engine uses it: chapati–LPG is flagged REVIEW — 161 g of gas recorded where every other fuel used 20–30 g. It's in the raw data and reproduced in the eCAP report, but it's almost certainly a transcription error (possibly cylinder weight difference misrecorded). I kept it as recorded rather than guessing a correction; if you can check the original field sheets, fix it at the source and rerun the script.

parameters.csv — every value carries source, as-of date, and a status flag: verified (checked against current EPRA/KPLC publications), monthly_update (the pass-through charges EPRA gazettes each month), update_locally (retail prices you or a field agent must survey), and fixed (physical constants). Key current values: base tariffs are still the 2023 schedule — lifeline 12.23, ordinary 16.45, high 19.08 KSh/kWh — because KPLC withdrew its proposed 2026–2029 tariff hike in June, and the government froze the base rates. June 2026 pass-throughs: FECC 3.14, forex 0.7154, WRMA 0.0142.

tariff.py — computes the effective all-in cost per kWh (base + pass-throughs, VAT, REP and regulatory levies). With June 2026 charges: lifeline ≈ 19.31, ordinary ≈ 24.42, high ≈ 27.60 KSh/kWh. Note these are well below the 22.84/27.78 the eCAP report used in October 2023 — electricity has gotten cheaper relative to that baseline, which strengthens the eCooking savings case and is worth a sentence in your report.

The structural finding: there's now a third domestic band (>100 kWh at 19.08 base) that didn't feature in the eCAP analysis, and bands are assigned on 3-month average consumption — the whole bill repriced, not tiered. tariff.py therefore computes the band from total projected consumption rather than accepting the user's current band, which handles the case where adding eCooking pushes a household across a threshold. That's the Step 4 tariff-interaction logic, already done.

Three things to update before validation: charcoal and ethanol prices are still 2023 values (flagged STALE — no current verified source; survey locally or check KOKO's current point price), the LPG figure is a Nairobi mid-range with a huge dealer spread (2,700–3,510 per 13 kg refill), and EPRA's July pump price review lands 14 July, which will move kerosene.